COMPLETE TUNING GUIDE FOR JEEP JL/JT

Updated 5/24/2026

This guide will walk you through EVERYTHING you need to know about having BMP Calibrations tune your Jeep.  If you have any questions about anything, feel free to email Brad at brad@bmpcalibrations.com at anytime.  You can also call or text Brad at 618-977-5844

COST BREAKDOWN

***Please note that 2021-2023 models were shipped with either GPEC2A OR GPEC5, check yours! PCM is located under the hood, typically next to the windshield washer fluid container or along the firewall, connectors on the side means you have a GPEC2A, connectors on the top means you have a GPEC5***

***These costs are assuming you do not currently own an MPVI device, credits, or a bypass device.  If you already own a Tazer or other type of SGW bypass cable, you do not need the HP Tuners Smart access cable.  These prices are worst case scenario for customers starting from zero***

| GPEC2A MODELS **PCM Connectors on SIDE of the PCM |  | GPEC5 MODELS **PCM Connectors on TOP of the PCM |  |
| --- | --- | --- | --- |
| 18-23 JL/JT 6MT | 18-23 JL/JT 8MT | 21-26 JL/JT 6MT | 21-26 JL/JT 8AT |
| TO BMP CALIBRATIONS | TO BMP CALIBRATIONS | TO BMP CALIBRATIONS | TO BMP CALIBRATIONS |
| $300 | $400 | $300 | $400 |
| HP TUNERS MPVI4 $400 |  |  |  |
| HP TUNERS DODGE SMART ACCESS CABLE $90 |  |  |  |
| HP TUNERS UNIVERSAL CREDITS ***One Time Purchase, $50 each*** |  |  |  |
| $100 | $300 | $200 | $400 |
| PCM UNLOCK SERVICE |  |  |  |
| Free with new MPVI4 and 6 credits, otherwise $250 |  | $250 |  |
| TOTALS |  |  |  |
| $1140 + tax | $1190 + tax | $1340 + tax | $1540 + tax |

FREQUENTLY ASKED QUESTIONS

Is tuning safe?

-YES, in many ways it is safer than factory tuning because we are datalogging and dialing in the tune for your exact environment and fuel quality.

Why BMP?

-I have over 5 years of hands-on tuning experience on a variety of platforms, with hundreds of vehicles tuned both locally and remotely.  I started with Ford Ecoboost platforms which is one of the most challenging platforms to master, dabbled in GM stuff, then put all of my focus on Jeep as there are very few people out there tuning Jeeps. I have taken every single available tuning and calibration course I can get his hands on and continue to learn new things as things come along.  Additionally, I am a former Mopar dealer technician and a Jeep dealer service manager, and to this day I still do a ton of side work repairing and maintaining vehicles.  Additionally, I have been a police officer for 13 years and continue to be a full-time police officer to this day and as such, I pride myself on a high level of integrity, doing what is right, thorough and quick communication, and attention to detail.  I am not just sending everyone cookie cutter email tunes, every single customer I tune for gets a fully custom and dialed in experience made just for their Jeep.

Will tuning void my warranty?

-No. Dealers and techs can’t tell you’ve been tuned outside of a “flash counter” deep within the WiTech software, but even bone stock vehicles have elevated flash counters with software updates, so that’s common.  P1400 DTC for Aftermarket Calibration is disabled in all my files, if you have a warranty repair the dealer would have to prove the tuning was the cause. If your PCM itself fails, that is the only circumstance where a warranty repair could possibly be denied, but this in and of itself is rare. Aftermarket warranties I can’t speak for, sorry.

Will my tune be lost if the dealer updates my software?

-Let me know after this happens and I will apply your existing tune changes to your new OS.

Can I purchase a spare PCM to swap?

-Yes, but do this through your dealer, making sure your old and new PCM have the same exact software update programmed on to them BEFORE sending one of them out for unlocking at HP Tuners, I would stay away from having HP Tuners create a clone (they offer this for GPEC2A) as I have seen issues with this before on 6MT models.

I have a check engine light after installing my unlocked PCM, is this normal?

-Completely normal and expected, clear DTCs with VCM Scanner and the CEL will go off, may have to do this twice, some codes will remain displayed until 2-4 drive cycles are completed.

My engine is running funny on first start up with the unlocked PCM, what happened?

-Fairly common, use VCM Scanner to clear DTCs and clear adaptives.  Then let the Jeep sit for 10-15 minutes and come back and it’ll run perfectly normal.

Will I pass emissions/inspection?

-EGR and emissions related parameters are left stock with my tuning and you should not have any issues, you can always flash back to stock for a few weeks before your inspection if you are concerned about it.

Can I come in person for tuning?

-Of course, but you will need to arrive with your already unlocked PCM, I do not offer PCM unlocking and that MUST be done through HP Tuners or your trusted local shop that has the ability to do it (many shops do GPEC2A, GPEC5 is HP Tuners only at this time).

Can I change my tire size and gear ratio without getting a new tune?

-Yes, this can be done with your MPVI4 using VCM Scanner at any time.

Why do I have to pay HP Tuners for credits after already purchasing a device and unlock from them?

-HP Tuners is a very large company that employs dozens of engineers that spend a lot of time and money in development for multiple makes and models, the credit system is how HP Tuners recoups their expense and time, they are a business after all.

Do I have to buy credits every time I want to change a tune?

-NO, credits are one time only attached to your MPVI and vehicle, once applied, you’ll never have to buy more for that vehicle unless you get a different MPVI device

BENEFITS OF TUNING – WHAT IS ALL DONE?

My tuning completely recalibrates the engine and transmission. On every base tune (the V1 file we start with), I revamp the variable cam function from the ground up, modify part throttle spark timing and wide open spark timing, make changes to torque management including removing throttle limiters, torque delivery mapping, throttle pedal mapping, transmission shift scheduling, transmission shift characteristics, auto stop start functionality, and more! My tunes result in 120-160 tables being modified to enhance your drive from the ground up. All these tables were individually tested and data logged one at a time during development, with thousands of miles of testing completed before launching the final product, all things within the PCM are tuned within established factory parameters and will not hurt or strain your vehicle.

The end result is a Jeep with much more usable power below 3000 RPM, a smoother powerband, a flatter torque curve, better shifting, less bogging down in down, more horsepower and torque all the way to redline, stop start disabled. Manual transmission vehicles will be much easier to control on clutch let off and will be much smoother to use.  Automatic transmission models will have the RPM right where you want it at all times – downshifts come much easier, engine won’t bog down in town.  GPEC2A models can have the cooling fans for the radiator come on much earlier (GPEC5 is still in development for that at this time).  On GPEC5 models with the infamous “stutter” or misfire feeling at low load low RPM (3-4 shift around 2000 RPM on automatic models, multiple other places this is felt on 6MT models), this issue is completely fixed with my calibration.  Your Jeep will drive the way it should have from the factory with my tuning.  In addition to all the above benefits, with BMP you get a truly custom experience, with a heavy emphasis on speedy customer service and response. I can’t stand unread emails/messages/texts/calls, and if I’m awake, I reply.  Simple as that.  Your Jeep will be data logged after getting tuned and this log is reviewed to ensure your Jeep is running as safely and as healthy as possible.  BMP Calibrations often fix things that your stock tune was doing wrong, like having too much part throttle timing, too little fuel, bad airflow management, and more.  Having a properly custom tuned vehicle with good data log review will make your vehicle more reliable, more efficient, and run how it should have from day 1.  On 87 octane, I regularly see minimum 10HP/10TQ gains, with a much better torque curve and higher torque coming in at a lower RPM.  On 93 octane, I have seen as high as 40 HP/30 TQ improvements, especially going from 87 on stock tuning to 93 with my custom tuning.  Horsepower and TQ figures vary from vehicle to vehicle depending on environment.  I monitor these numbers in datalogging and can let you know where your vehicle stands as we go through the process! Those numbers don’t even come close to explaining the dramatic drivability improvements you will experience with a good custom tune.

STEP 1.1 - BMP CALIBRATIONS TUNE PURCHASE

Purchase my tune services using the links below

Trans Only Tune for 8 Speed Auto Owners - $200

https://www.paypal.com/ncp/payment/7WYK8NQ6J6G5J​

6 Speed Models - Complete Engine Tune - $300

https://www.paypal.com/ncp/payment/4G48PECNZL492​

Automatic Models - Complete Engine and Transmission Tune - $400

https://www.paypal.com/ncp/payment/V2CAFGS6FD3DS​

STEP 1.2 – PURCHASES FROM HPTUNERS.COM

****If coming in person for tuning, you technically do not need your own MPVI4 and access cable as we can use mine, potentially saving you $500, but you will still need to have your PCM unlocked at HP Tuners as I do not provide that service.  I highly recommend purchasing and using your own MPVI4 and access cable, because if you ever want to return to stock or change something, you’ll be able to without driving back to me******

This is the expensive part; this money goes to HP Tuners directly and not BMP Calibrations LLC.

- Go to www.hptuners.com.

- Create an account (free) and login to it.

Note that your MPVI4 and access cable will NOT ship until they are ready to send your PCM back to you, if you want your MPVI4 to come beforehand, you need to order it separately from your PCM unlock.

- Add an MPVI4 ($400), along with the number of credits you need, to your cart.

- Credits are $50 each, this is a one-time purchase per vehicle tuned, meaning, once you flash your credits to your VIN, you can tune and flash with your MPVI4 as much as you want.

- 4 credits are needed for GPEC5 PCMS (connectors on the top of the PCM),

- 2 credits are needed for GPEC2A models (connectors on the side of the PCM)

- 4 additional credits are needed for TCM on automatic models.

- Add a Dodge Smart Access Cable for 2018+ models ($89) to your cart

-If you already have a bypass cable from  JScan or Tazer, you can skip this.

- Add PCM Unlock Service to your cart (skip if you're only tuning TCM)

GPEC5 - https://www.hptuners.com/product/dodge-pcm-service-gpec5/

GPEC2A – https://www.hptuners.com/product/dodge-pcm-services/

-For GPEC2A users, combining this with a new MPVI4 and 6 credits will make your PCM service FREE. Sorry GPEC5 users have to pay full price.

-Discount should be applied at check out automatically for GPEC2A, saving you $250.

-No need to buy additional PCM, clone PCM, or upload HPT file, just buy the service by itself and ship your existing PCM.

STEP 2: PCM REMOVAL FOR SHIPPING TO HP TUNERS

- Disconnect negative battery terminals for both aux and main battery (on top of negative post on main battery, unbolt each line separately and isolate to prevent issues or shorting potential).

- Your PCM is located next to your windshield washer fluid on the driver’s side under the hood.  Make sure the area is free of dirt, debris, etc before removing PCM connectors. Connectors on top means you have a GPEC5, connectors on side means you have a GPEC2A.

- Carefully remove PCM connectors, there are two, there is a push button to release the large lever, once released the lever pulls back and helps the connector separate from the PCM.

- Remove the bolts holding the PCM in, should be 4 10MM bolts.

- Keep track of where the ground strap, if any, were located, they must be installed when going back in.

- Package PCM and ship to HP Tuners

- HP Tuners, LLC.
ATT: Dodge Upgrade Service
[Insert Your Order Number Here]
700 Eastwood Ln
Buffalo Grove, IL 60089

-Form should have been included in order email, but if not, print one and fill it out to place in box with your PCM.  Form can be found at https://files.hptuners.com/pub/2024/10/Dodge-PCM-Upgrade-Service-Shipping-Form-v3.pdf

-Now, wait for HP Tuners to send your unlocked PCM back to you, this typically takes 5-7 business days, but sometimes as fast as 3-4 if you overnight UPS or are close to Illinois.

STEP 3: PCM AND ACCESS CABLE INSTALL

- Install PCM reverse order of removal, be sure battery is still disconnected, don’t forget any ground straps! Be careful not to bend the pins

- Install access cable, here is a guide -

https://files.hptuners.com/pub/2025/05/2018-Dodge-Smart-Access-Cable-Instructions-v1.pdf

- Install negative battery cables again, a spark is normal.

STEP 4: FIRST START UP AND READ

*If you have already started your Jeep, you will notice a check engine light, don’t worry about it! It’ll go away after clearing codes and if not it’ll go away after we tune*

- Grab your PC, MPVI4, and included USB-C cable.

- Go back to hptuners.com and download the LATEST BETA, install it, it will install over your existing BETA

- Open VCM Scanner (yellow lightning bolt icon)

- Plug your MPVI4 into your OBD2 port, then plug your USB-C cable into it and your PC.

- Turn your key to the “ON” position without starting (two pushes on the button with foot off the brake, all dash lights should come on)

- Click the blue vehicle icon to “Connect to Vehicle”

- Click the orange check engine light for “Diagnostics and Info”

- Read DTCs, various codes relating to throttle body and other circuits is normal.

- Clear DTCs.

- Start engine and ensure CEL is off, if it is on, go back to step 6 again and try again. DON’T PANIC.  If your CEL does not go off, we will fix it in the next steps, nothing is wrong! You may still have DTCs displayed as permanent or pending, this is normal!!

- Turn Engine Off

- Close VCM Scanner and open VCM Editor (Grey wrench icon).

- With Key On but the Engine Off, Click the “Read Vehicle” button (Black with green arrow).

- Click “Gather Info”, it will determine what PCM and TCM you have

- If your OS is newer and not defined by HP Tuners, these boxes will be empty or blank, you just have to manually select your PCM and TCM.  Scroll and find Stellantis GPEC2A or GPEC5 as needed.  For TCM, usually you have to select Dodge ZF8HP2.

- Click “Read”.

- When the Read is done, SAVE THAT FILE!!!! THIS IS YOUR STOCK/AS FOUND READ.

- Email me that .HPT read file.  I will build your tune from that.

STEP 5: FLASHING THE NEW TUNE

Follow these steps to write a tune back to the vehicle:

-Plug the interface cable into the vehicle's OBD-II port. Connect the other end to your PC's USB port.

-Open VCM Editor. Open the tune file I sent you.

-Select Write Vehicle from the Flash menu.

NOTE: The first time you attempt to write to each vehicle, you will be prompted to purchase a license for that vehicle, if already purchased credits, you apply them at this time..

-Turn the vehicle ignition to the ON (or RUN) position, but do not start the engine.

-Make sure the doors are closed and all accessories are turned off. CAUTION: Do not use any vehicle accessories, including the door locking mechanism, during reading or writing procedures.

-Wait 10 to 15 seconds.

-Make sure ‘write calibration’ is selected for the PCM and TCM if you’re an automatic.  Then hit the Write button.

-Follow the prompts on the screen.

DATALOGGING INSTRUCTIONS

-After plugging into your vehicle and opening VCM Scanner, click the blue car button to ‘connect to vehicle’, go to the Channels tab (on the left) and click the folder icon to open a channel configuration. Open the configuration file I provided you via email.

-After the config is loaded, click ‘vehicle’ at the top menu, then click ‘Repoll for for Supported Parameters’.  This will ensure all data I need gathered actually populates.

-If a bunch of channels are missing or you had BLANK boxes in VCM Editor when you did your read, we need to change your vehicle profile in VCM Editor. To do this, click “vehicle” at the top menu, then click “Vehicle Profiles” and select “vehicle profile editor”.  Select your vehicle/VIN.  To the right you will see blank boxes for “User OS”.  On the ECM part (the top box) in the User OS box type the following OS number > 68729562AA and for the TCM part (auto models only,the bottom box) type the following OS number > 68671028AA.  Click the save button and save that profile somewhere.

Now your datalog channels should work!  Load the channel configuration file I sent in the channels and do “Repoll for supported parameters” in the "Vehicle" drop down again and everything should be good.

-You are ready to datalog now, hit the red record button.  Make sure that all the channels in your channel list are solid and not greyed out and that they are actually populating data, if they arent’t, stop recording and try again. Sometimes this takes a few tries.

While recording and ensuring all the channels are pushing out a value, go for a 5-10 minute drive, I need at least 1 wide open throttle pull, as well as mixed city driving.  Drive with a variety of aggressiveness and use the whole range of the pedal and engine to get various loads and areas within the tune datalogged.  The most critical is wide open throttle.  For automatic models, considering going into manual mode like 2nd or 3rd gear, and mashing the throttle from 1500 RPM all the way to redline on a highway or back road somewhere to simulate a dyno pull.  If you are datalogging to show me a particular issue or area you want changed within the tune, only record those areas.

-Hit the save button up top after the datalog is done.  Send me that file.  Rinse and repeat until I say it looks good.  Ask for any revisions you want when sending the datalog.

Here is a deeper breakdown with graphics for reference………………..

Hp Tuners Data Logging Instructions

‌

VCM Scanner

The VCM suite you downloaded contains both "VCM Editor" and "VCM Scanner." For the rest of this guide we will be using "VCM Scanner"

Open "VCM Scanner" and select the icon shown to the right:

From the drop down menu select "Open Channel Config" and navigate to where you saved the provided channel config file, click on it so its highlighted and select "open".

You should see the channels list populate on the left side of "VCM Scanner" as shown below:

Logging

Now that our channels are set up we can plug the MPVI into your OBD2 connector, and with the supplied USB-C cable connect your laptop.

Start the truck and select "Connect to Vehicle" from the toolbar at the top:

Once connected and in a safe place to perform your data log, select the Red Circle, "Start Scanning" button to begin recording.  If you have a specific problem, attempt to duplicate the issue while logging. Logs can be short and sweet, no need for a 10-15 minute log unless specifically asked.  When you are finished with your log, select the square "Stop Scanning" button from the toolbar, then "Save Log File As" give the log a short name describing it and save it somewhere you can find it to send it in for review. Starting a new log before saving the last will result in that data being lost. If the log was successful and you want to keep the data, be sure to save it before starting a new one. Once your log or logs are saved, email them to me at brad@bmpcalibrations.com

If you have any questions or issues, email me or call me at 618-977-5844.